Document camera fix

Hello Monica,

Thank you for contacting SMART Technologies.

I understand that you are unable to obtain an image with your SMART Document Camera within the SMART Notebook(Tm) collaborative learning software.

Please follow the steps below to resolve your issue;

Click on the Start button and then right click on My Computer and select Properties

Select the Hardware Tab and then click the Device Manager button

Locate the entry for the Camera.  Should be under Universal Serial Bus Controllers, but not always. You might have to search to find it under another heading such as disc drives, etc.

Right click on the entry and select Update Driver

Select the Yes This time only radio button

Click next and then select the Install from a list or specific location

radio button

Click next

Make sure Search for the best driver in these locations is selected and Check Include this location in the search

Browse to that file path c:\Program Files\SMART Technologies\SMART Product Drivers\SMARTDocumentCamera\Gen1\ OR Document Camera 330 for the latest model

Simply reply to this email with the outcome of these steps.